ESA 68

A COST EFFECTIVE & VERSATILE MICROPROCESSOR TRAINER

ESA 68 is an extremely powerful Microprocessor trainer based on the popular Motorola 6809 eight bit microprocessor. It can be used as a flexible instructional aid in academic institutions and R&D labs.

MAIN FEATURES:

- ESA 68 can be operated either from on-board keyboard or from a CRT terminal / host computer through its RS 232 C interface.

- Keypad and serial monitor programs support the entry of user programs, editing, debug facilities like a single stepping and full speed execution of user programs.

- 32K bytes of CMOS static RAM is provided with battery back-up facility . Total on board memory expandable upto 56K bytes.

- Operates both in keyboard / serial mode on a Single +5V power supply.

- On-board general purpose prototyping area for user expansion.

- Allows multi-processor system design by supporting DMA / BREQ* signals.

- One user defined function key.

- It has on board spectra-strip type ribbon cable connectors for easy system expansion and interfacing.

- All interrupts except NMI* are available to the user. If single-step function is not required, NMI * can also be used by the user.

ACCESSORIES (Optional):

- Power supply: 5V @ 3.0A, +/- 12V @ 100mA, -12V @ 100mA, +30V @ 100mA  housed in ABS Plastic enclosure.

- Interface Modules for training: Calculator type / Keyboard, Elevator, 7 Segment Display, ADC using DAC, Dual Slope ADC, Dual DAC,  8 bit 16 Channel  ADC, Logic Controller, RTC, Tone Generator, Stepper Motor, Opto Isolated Input, Opto Isolated Output, Relay Output, LCD, etc.,

- Rechargeable NI - CD Battery Cells ( Two) for battery backup of RAM.

- XT 68 driver software for PC compatible systems including RS 232 C Cable.

SPECIFICATIONS:

| CENTRAL PROCESSING UNIT | MC 6809 operated at 4 MHz |
| --- | --- |
| MEMORY | Three 28-pin JEDEC sockets provide 56K bytes of memory: EPROM:24K bytes using 2764 (8K) and 27128 (16K) one each. Out of 24K bytes of memory firmware is supplied in 2764. RAM:32K bytes of RAM using 62256 with optional battery backup, 8K bytes of memory is used for I/O addressing. |
| INTERFACE SIGNALS | CPU BUS:Buffered TTL bus compatible signals brought out to a 40 pin ribbon cable connector. Parallel I/O:32 lines of 6821 PIA, TTL compatible signals brought out to ribbon cable connectors. Serial I/O:RS 232 C compatible with standard MODEM control signals through on board 9 pin D-type female connector. |
| PERIPHERALS | 8279-5:To control 32 keys keypad and 6 digit 0.5 inch 7 segment LED display. 6840:Programmable Timer Module PTM - provide 3 programmable interval timers. Timer 1 is used for implementing single stepping facility , Timer 2 and Timer 3 is available to the user. 6850:Asynchronous Communication Interface Adapter ( ACIA ) , For serial communication supports all standard baud rates from 150 to 19,200, selectable through jumpers. 6821:(2 Nos) Peripheral Interface Adapter (PIA) provides 32 programmable I/O lines. |
| POWER SUPPLY | 5 V, @ 750 m A in Keyboard Mode |
| DIMENSIONS | With ABS Plastic moulded cabinet. L X B X H: 365 X 275 X 55mm Weight: 1100gms(Aprox.) |
| DOCUMENTATION | User's Manual |