ESA 80

A COST EFFECTIVE MICROPROCESSOR TRAINER

ESA 80 is a versatile and cost effective Microprocessor Trainer based on the popular Z 80 CPU, supported by a powerful monitor program and a variety of interfaces. ESA 80 can be used as a complete single board microcomputer, as an instructional aid in colleges and as a development tool in research institutions, industries and R&D labs.

MAIN FEATURES:

- ESA 80 can be operated either from on-board keyboard or from a PC compatible system through its RS 232 C Port.

- User can enter programs, debug them through Single Step, Breakpoint facilities and also run them at full speed.

- Provision for on-board ADC 0809.

- Facility to edit programs through INSERT, DELETE, and BLOCK MOVE commands.

- Provision for battery backup to RAM.

- Three on-board ribbon cable connectors for easy system expansion.

- On-board audio Cassette Interface.

- On-board general purpose PCB, for user expansion.

ACCESSORIES (Optional):

- Power supply: +5 V @ 3.0A; +12V @ 250mA; -12V @ 100mA; + 30V @ 100mA housed in ABS Plastic enclosure.

- Interface Modules for training: Calculator typr Keypad, Elevator, 7 Segment Display, ADC using DAC, Dual Slope ADC, Dual DAC, Logic Controller, Crystal Clock Divider, Traffic Lights, Stepper Motor, Numerical Printer, RTC, LCD, etc.

- NI - CD Cells for power backup to RAM.

- Driver Software for communication with PC Compatible Systems including RS 232 C Cable.

SPECIFICATIONS:

| CENTRAL PROCESSING UNIT | Z 80 operates at 1.78 MHz |
| --- | --- |
| MEMORY | Provision for 64K Bytes in combination of ROM / RAM 16K Bytes of Monitor Program (27128) and 32K Bytes of RAM (62256) are supplied. Remaining 16K Bytes of memory (RAM / ROM) area is for user expansion. |
| INTERFACE SIGNALS | Parallel I/O:40 lines (24 of 8255 and 16 of PIO ) brought out to two 26 pin ribbon cable connectors. Serial I/O:RS 232 C with standard MODEM control signals through on board 9 pin D-type female connector. Cassette Interface:Audio tape interface signals provided via. two audio jacks. Interrupts:NMI is used by the system for implementing the Single Step facility. INT is available to the user. |
| PERIPHERALS | 8255-5:(Two numbers) One 8255 is used to control the 36 keys Keypad, 6 digit 0.5" seven segment LED display and Audio Cassette Interface. The other 8255 is available to the user, providing 24 Programmable I/O lines. 8253:3 programmable interval timers - Timer 0 is used for generating baud clock and Timer 1 & 2 is available to the user. 8251A:For serial communication. Supports all standard bauds from 110 to 19200 ( Baud is selected through on board DIP switch) Z 80 PIO:Provides 16 Programmable I/O lines with hand shake feature. ADC 0809:Optional on-board 8-bit, 8 channel A/D converter. Precision +5V on-board reference voltage generation. |
| BUS EXPANSION | Buffered TTL compatible bus signals brought out to 50 pin ribbon cable connector for easy expansion. |
| POWER SUPPLY | +5V @ 770mA in keyboard.+5V @ 640mA in serial. +/- 12V @ 25mA in serial |
| DOCUMENTATION | 1)ESA 80 User's Manual. 2)Keypad Monitor Listing |
| DIMENSIONS | With ABS Plastic moulded cabinet. L X B X H: 365 X 275 X 55 mm Weight:1300 gms (Aprox.) |