ESA ICE-1

A COST- EFECTIVE IN-CIRCUIT EMULATOR

ESA ICE - 1 is a cost effective and versatile development/debugging tool, extremely useful for Educational Institutions, R&D Labs and Industries. It has an RS 232 C port through which it can be interfaced with Host computer systems / CRT terminals. ESA ICE 1 supports a variety of popular 8 bit processors and provides a powerful command set for the development /debugging needs.

MAIN FEATURES:

- Supports the popular processors 8085, Z-80, 6502 and 8048 family.

- EPROM resident firmware.

- Operation upto the full speed of the Target Processor.

- Target processor retains its entire Memory and I / O space.

- Emulation memory of 8K Bytes of static RAM. Memory, Write Enable/Disable and Base Address selection through DIP switch.

- Clock source from Target/ESA ICE-1.

- Single character commands. HELP command to list the syntax for all the commands.

- Provision for enabling/disabling hardware control signals to the processor.

- Support for Hardware debugging through CYCLE STEP, TRANSFER / TEST, INPUT, OUTPUT, ENABLE, DISABLE Commands.

- Support for software debugging through INSTRUCTION STEP, BREAKPOINT, REGISTER and TRACE commands.

- Breakpoint supports loop count and cycle qualification.

- Trace Buffer of 256 cycles.

- Resident one-line Assembler and Disassembler useful for field patch work and also when interfaced with only CRT terminal.

- Optional communication package XTICE 1 allows interfacing with IBM PC/XT/AT compatibles. Support for Download / Upload of INTEL Hex files.

- Provided with personality module for one processor. Additional modules for other processors are optional.

- ESA ICE 1 is housed in a light-weight enclosure as it works with an external DC power supply. This feature allows ESA ICE 1 to be located nearer to the target system.

COMMAND SUMMARY FOR 8085 PERSONALITY MODULE:

ASSEMBLY	A	loc

BACKWARD TRACE 	B	tri [a1 [a2 [q1...qn]]]

CYCLE STEP	C

DISABLE	D	T | H | I

ENABLE	E	T | H | I

FORWARD TRACE	F	tri [a1 [a2 [q1...qn]]]

EXECUTION	G	[a]

BREAKPOINT	H	[a[c]]

INPUT	I	port[c[time]]

JUMP	J	a

LIST TRACE	L	[c1[c2]]

MEMORY	M	a[a[d]]

OUTPUT	O	port d1 [d2[d3[d4]]]

REGISTER	R	-|A|B|C|D|E|H|L|M|P|S

INSTRUCTION STEP	S	[c]

TRANSFER / TEST	T	a1 a2 S|M|a3

UPLOAD	U	a1 a2 _|T|I

RESET	X

DISASSEMBLY	Z	a1 [a2]

DOWNLOAD	:	(INT), /(TEK)

HELP	?

ATTENTION	!

DOCUMENTATION SUPPLIED:

ESA ICE-1 User Manual

OPTIONS:

- Additional Personality modules for 8085, Z-80, 6502 and 8048 family.

- Power Supply +5V DC,1.2A +/- 12V DC, 100mA.

- RS 232 C Cable.

- XTICE 1 Communication Package for IBM PC/XT/AT Compatible systems.

### XTICE: DRIVER SOFTWARE FOR PC COMPATIBLE COMPUTER

This software package establishes a communication link between In-Circuit Emulator ESA ICE - 1 and PC compatible host system. The lnk is made over the RS 232 C communication port of PC (COM1 / COM2) and the serial por of ESA ICE - 1. This user friendly menu driven software package (supplied in a 3 1/2" floppy) supports uploading/downloading of Intel HEX file format to/from the PC compatible system. Thus the extensive software development facilities of the system ae made available to user.

MAIN FEATURES:

- This package links the In - Circuit Emulator ESA ICE - 1 with any PC compatible system over RS 232 C communication port (COM1/COM2) at variable baud (110 to 9600), 8 data bits, 2 stop bits, no parity.

- The download command allows the user to load the object code in INTEL HEX format generated by cross assembler/cross compilers from the PC compatible system to ESA ICE - 1.

- The upload command an be used to save the memory contents of ESA ICE - 1 (in INTEL HEX format ), in the storage devices of PC.

- Command file facility.

- Command line editing and command recall facilities.

- Switching between DOS and Driver software environment.

- Accepting DOS commands in Driver software environment.

- Context sensitive help.